PRODUCT  DATA  SHEET

###### ELECTRO-SOLV “S”

ELECTRIC / ELECTRONIC

DEGREASER

PRODUCT  DESCRIPTION

Electro-Solv “S” is a new non-ozone depleting degreaser for electrical and electronic parts. Electro-Solv penetrates quickly, leaves no residue, is fast evaporating, non-conductive and safe for use on metals and plastics.  It is formulated specifically for degreasing electrical and electronic parts, equipment and other sensitive mechanisms.  Electro-Solv penetrates into cracks and crevices to remove oils, grease, grim, flux, and other contaminants.

Use on tuners, tape heads, connectors, contacts, office equipment, printed circuits, switches, relays, controls, semi-conductors, breakers, and numerous applications.

TYPICAL  CHARACTERISTICS

DIRECTIONS:  Can temperature should be 18ºC (65ºF) or warmer.  (WARNING:  Never use direct heat to warm aerosol cans!  Only warm water should be used!)  Insert plastic extender tube into spray tip for pinpoint control and elimination of over spray.  Spray electrical equipment and allow 30 seconds for soil and other deposits to soften.  Flush away soil with second spraying.  Allow to dry thoroughly before restarting or reusing equipment.

Sentinel’s “S” lines of spray lubricants are non-ozone depleting products specially formulated for industry.

Sentinel’s “S” line contains no CFC’s, HCFC’s, Methyl chloroform, or any other material known to affect the stratospheric ozone.

Appearance			               Clear liquid

Flash Point, ºC (ºF)			                    < -17 (0)

Specific Gravity, g/cc @ 60/60ºF			                           0.66

Weight, lbs./gal @ 70ºF			                             5.5

ODC Content, % wt			                          None

Surface Tension, dynes/cm @ 77 ºF			                            17.5

Dielectric Strength, KV			                               35

Propellant			              	       CO2

USDA Classification			             	        H-2

Kauri-Butanol Value			                		   28

Net Weight per 20oz Aerosol Can			                14oz. (397g)

PIN #			                          01005

TO ORDER or FOR ADDITIONAL INFORMATION