PRODUCT  DATA  SHEET

G

###### CITRA-SOLV

ALL NATURAL CLEANER

AND DEGREASER

PRODUCT  DESCRIPTION

Sentinel’s Citra-Solv is a heavy duty cleaner and degreaser that will easily remove dirt, oil and grease from surfaces.  Citra-Solv is all natural, biodegradable, water risible, and non-toxic.

TYPICAL  CHARACTERISTICS

Citra-Solv is used to clean knives, saw blades, tools equipment, tables, painted surfaces, conveyers, hard surfaces, machinery, industrial equipment, materials handling equipment, and electric motors.

Specific Gravity, g/cc @ 60/60°F				   0.85

Flash Point, °C (°F)				       46 (115)

Boiling Point, °C (°F) 	 		     	  154 (310)

Acid Number, mg KOH/g					 0.14

Kauri Butanol Value 					 62.7

Weight/ lbs/gal @ 21°C (70°F)					7.1

Refractive Index				 1.4711

Odor				 Strong Citrus

Color				Clear to Yellow Cast

PIN #    											02320

TO ORDER or FOR ADDITIONAL INFORMATION

TECHNICAL DATA

###### CITRA-SOLV

ALL NATURAL CLEANER

AND  DEGREASER

DIRECTIONS:

TO CLEAN & DEGREASE SURFACES:  Dilute 4 oz. Into 1 gallon of warm water.  Apply solution generously to surface to be cleaned.  Let stand for 5 minutes.  Rinse thoroughly.

FOR ODOR CONTROL:  Dilute 2 oz. Into 1 gallon of cold water.  Apply through a sprayer to odor source.  Results in immediate fresh citrus scent.

TO UNCLOG DRAINS:  Pour 8 oz. directly into drain.  Let stand for 1 hour or more.  Flush with hot water.  For disposal, use same procedure.

USED TO:

Degrease Trucks								Remove Decals

Unclog Grease  								Clean Drains

Remove Printer’s Ink							Remove Adhesive

Remove Tire Marks							Clean Motors

Remove Soap Scum							Clean Equipment

De-tar Vehicles & Loosen Chewing Gum from Carpets & Hard Surfaces (other than plastic)

Cleans Fuel Oil Deposits from Metal Surfaces

100% Organic Formulation.  1 step degreasing and deodorizing.  NON-CORROSIVE. Contains no Petroleum Distillates, Organic Halides or Alkalies – Will strip grease and heavy oil deposits from virtually any surface.  Emulsifies instantly in water to penetrate and liquefy many different types of resins and greases.

CAUTION:  KEEP OUT OF REACH OF CHILDREN

Combustible – Keep away from heat and open flame.  Do not take internally.  Avoid contact with eyes.  In case of contact, immediately flush eyes with plenty of water for at least 15 minutes.  If irritation persists, seek medical attention.  If swallowed, consult a physician immediately.  May be harmful to plastics or painted surfaces.  TEST SURFACE IN AN INCONSPICUOUS AREA BEFORE USING.  Because it is powerful grease solvent, it will defat skin.  Use of rubber gloves are recommended to avoid chapped skin.  Keep container tightly closed when not in use.  For Commercial and Industrial Use only, or for sale to, use and storage by service persons.

USED  IN:

Sewage Plants									Schools

Service Stations									Restaurants

Garbage Trucks									Office Buildings

Automotive Plants									Apartments

Theaters										Nursing Homes

Locker Rooms									Hospitals

Rest Rooms									Commissaries

TECHNICAL  DATA

###### CITRA-SOLV

ALL NATURAL CLEANER

AND DEGREASER

COMPATIBILITY  CHART

CPVC										No effect-Excellent

Epoxy										No effect-Excellent

Polypropylene									Moderate effect-Fair

Polyethylene									Moderate effect-Fair

PVC										Minor effect-Good

Cycolac (ABS) (Reg TM Borg-Warner Co.)						Minor effect-Good

Phenolic										No effect-excellent

Nylon										No effect-excellent

Noryl (Reg TM General Electric Co.)							UNKNOWN

Delrin (Acetal) (Reg TM E.I. du pont de Nemours & Co.)					Severe effect-Not Recommended

Ryton to 200°F (Reg TM Phillips Petroleum Co.)						UNKNOWN

Kynar (PVDF) (Reg TM Pennwalt Corp.)						No effect-Excellent

Teflon (Reg TM E.I. Dupont de Nemours & Co.)						No effect-Excellent

Stainless Steel (316)								No effect-Excellent

Stainless Steel (304)								No effect-Excellent

Stainless Steel (440)								No effect-Excellent

Titanium										No effect-Excellent

Carpenter 20									No effect-Excellent

Cast bronz									No effect-Excellent

Aluminum									No effect-Excellent

Hastelloy C									No effect-Excellent

Carbon/ceramic									No effect-Excellent

Ceramagnet A									No effect-Excellent

Viton (Reg TM E.I. du pont de Nemours & Co.)						No effect-Excellent

Burna N										No effect-Excellent

Neoprene										Moderate effect-Fair

Nitrile										No effect-Excellent

Natural Rubber									Moderate effect-Fair

Hypalon (Reg TM E.I. du pont de Nemours & Co.)					UNKNOWN

EPDM										UNKNOWN

Kel-F (Reg TM 3M Company)							UNKNOWN

Tygon (Reg TM Norton Co.)								Moderate effect-Fair

Silicone										Severe effect-Not recommended

Ceramic										No effect-Excellent

Carbon/graphite									No effect-Excellent

Recommended Gasket Material:	Man-holes use Viton®

Pump parts use Teflon®

O-Rings use Fluro-Silicon®

Rubber gaskets must be periodically replaced

TO ORDER or FOR ADDITIONAL INFORMATION