SAFETY DATA SHEET

Citra-Solv

Revision B

Effective Date: 11/01/2018

Regulation: 1907/2006/EC, In accordance with the provisions of Article 41, Industrial Safety & Health Act.

OSHA Hazard Communication Standard (29 CFR 1910.1200).

1. IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

  - PRODUCT IDENTIFIER	: Material Name: Citra-Solv			Product Code	: 02320

  - RELEVANT IDENTIFIED USES OF THE SUBSTANCE OR MIXTURE AND USES ADVISED AGAINST

Product Use		: Citrus Cleaner Degreaser

Uses Advised Against	: This product must not be used in applications other than those 					                recommended in Section 1, without first seeking the advice of the supplier.

  - DETAILS OF THE SUPPLIER OF THE SAFETY DATA SHEET

Manufacturer/Supplier	: Sentinel Lubricants Inc.

15755 NW 15th Ave

Miami, FL 33169

Telephone			: Marketing Technician Department

1(800) 842-6400, (305) 625-6400

Fax                                          : (305) 625-6565

Email Contact for Safety Data Sheet	: info@sentinelsynthetic.com

1.4 EMERGENCY TELEPHONE NUMBER	: INFOTRAC – 1.800.535.5053     Contract #107464

International – 352.323.3500

2. HAZARDS IDENTIFICATION

2.1 HAZARD CLASSIFICATION 	: Flammable Liquids – 3, Skin Irritation – 2, Eye Irritation – 2

2.2 SIGNAL WORD			: DANGER

2.3 HAZARD PICTOGRAMS 		:

2.4 HAZARD STATEMENTS	: Flammable liquid and vapor. Causes skin irritation. May cause an allergic reaction. Causes eye irritation. May cause respiratory irritation. May be fatal if swallowed and enters airways. Very toxic to aquatic life with long lasting effects.

2.5 PRECAUTIONARY STATEMENTS

General Prevention	: Keep out of reach of children. Do not handle until all safety precautions have been read and understood. Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. Wash hands thoroughly after handling. Use only outdoors or in a well-ventilated area. Wear protective gloves, protective clothing, eye protection and face protection. Avoid release into the environment. Keep container tightly closed. Use only non-sparking tools. Take preventive measures against static discharge. Ground/bond container and receiving equipment.

Response	: If swallowed, immediately call a POISON CENTER or doctor. If on skin, wash with plenty of water. Take off contaminated clothing and wash it before reuse. If skin irritation occurs, get medical attention. If inhaled, remove person to fresh air and keep comfortable for breathing. If in eyes, rinse cautiously with waster for several minutes. Remove contact lenses if present and easy to remove. Continue rinsing. Do not induce vomiting. Call a POISON CENTER or doctor if you feel unwell. Collect spillage.

Storage	: Store in a well-ventilated place. Keep container tightly closed. Keep cool.

Disposal	: Dispose of contents, container in accordance with local regulations.

3. COMPOSITION/INFORMATION ON INGREDIENTS

3.1 INGREDIENTS

| Chemical Identity |  | CAS |  | % wt Range |
| --- | --- | --- | --- | --- |
| D-Limonene | Citrus Terpenes | 5989-27-5 |  | 75-99 |
| Nonylphenol polyethylene glycol ether | NP-6, NP-9 | 127087-87-0 |  | 5-25 |

As per paragraph (i) of 29 CFR 1910.1200, formulation is considered a trade secret and specific chemical identity and exact percentage of composition may have been withheld.

4. FIRST AID MEASURES

4.1 DESCRIPTION OF FIRST AID MEASURES

General Information	: If exposed or concerned, seek medical advice or attention.

Inhalation	: Remove to fresh air. If respiratory irritation, dizziness or nausea occurs, seek medical attention.

Skin Contact	: Remove with soap and water, rinsing and repeating for 15 minutes. Use skin cream to counter any resulting dryness. Consult a physician if irritation continues. If large skin area is affected, remove contaminated clothing.

Eye Contact	: Immediately flush with clear water for at least 15 minutes, including under the eyelids. Consult a doctor.

Ingestion	: Do not induce vomiting! Immediately have the victim drink plenty of water. Keep airway free. Contact a physician. Never give anything by mouth if victim is rapidly losing consciousness, unconscious or convulsing.

Self-protection

of the first aider	: Wear adequate personal protective equipment based on the nature and severity of the emergency.

4.2 INDICATION OF ANY IMMEDIATE MEDICAL ATTENTION AND SPECIAL TREATMENT NEEDED

Note to Physician	: Treat symptomatically.

5. FIRE FIGHTING MEASURES

5.1 EXTINGUISHING MEDIA	: Use alcohol resistant foam, carbon dioxide, dry chemical or water spray when fighting fires. Do not direct a water stream directly into the hot burning liquid.

5.2 FIRE OR EXPLOSION HAZARDS	: This material may be ignited if preheated to temperatures above the flash point in the presence of a source of ignition. Empty containers that retain product residue (liquid, solid/sludge or vapor) may be dangerous. Do not pressurize, cut, weld, braze, solder, drill, grind or expose container to heat, flame sparks, static electricity or other sources of ignition. Any actions can potentially cause an explosion that may lead to injury or death.

5.3 HAZARDOUS COMBUSTION PRODUCTS	: Oxides of carbon (CO, CO2), Acrid fumes.

5.4 ADVICE FOR FIREFIGHTERS	: Do not enter fire area without proper protection including self-contained breathing apparatus and full protective equipment. Fight fire from a safe distance and a protected location due to the potential of hazardous vapors and decomposition products. Use appropriate methods for the surrounding fire.

Protective Equipment	: As with any fire, wear SCBA pressure-demand, MSHA/NIOSH approved and full protective gear.

6. ACCIDENTAL RELEASE MEASURES

6.1 PERSONAL PRECAUTIONS, PROTECTIVE EQUIPMENT AND EMERGENCY PROCEDURES

For non-emergency personnel	: No action should be taken by non-emergency personnel without suitable training. Evacuate surrounding areas. Keep unnecessary and unprotected personnel from entering. Do not touch or walk through spill. Remove ignition sources and provide adequate ventilation if it is safe to do so.

For emergency responders	: Use personal protection as recommended in Section 8. Observe precautions provided for non-emergency personnel.

6.2 ENVIRONMENTAL PRECAUTIONS	: Use appropriate containment to avoid environmental contamination. Prevent from spreading or entering drains, ditches or rivers by using sand, earth or other appropriate barriers.

6.3 METHODS AND MATERIAL FOR CONTAINMENT AND CLEANING UP

Cleanup Procedures	: Remove sources of ignition and use non-sparking equipment. Soak up material with inert absorbent and place in safety containers for proper disposal.

Prohibited Materials	: Combustible adsorbent material such as sawdust, use of equipment that may cause sparking.

6.4 REFERENCE TO OTHER SECTIONS	: For guidance on selection of personal protective equipment see Section 8 of this Safety Data Sheet. For guidance on disposal of spilled material see Section 13 of this Safety Data Sheet.

7. HANDLING AND STORAGE

7.1 PRECAUTIONS FOR SAFE HANDLING

General Handling Precautions	: KEEP OUT OF REACH OF CHILDREN. Avoid prolonged or repeated eye, skin or clothing contact. Avoid breathing of vapors or mists. Avoid use around open flames or sources of ignition. Use only with adequate ventilation, opening doors or windows to achieve cross-ventilation. Wash hands after use. Do not take internally. Industrial use only.

Hygiene Recommendations	: Do not eat, drink or smoke when using this product. Wash hands thoroughly after use. Remove contaminated clothing and protective equipment before entering eating or smoking areas.

7.2 CONDITIONS FOR SAFE STORAGE

Storage Requirements	: Storage of individual cans should be done in a cool dry place. Ensure can is in a secure place to prevent knocking over and accidental rupture. Keep container tightly closed in a cool, well-ventilated place away from ignition sources. Keep away from food and drink. Ground and bond metal containers for liquid transfer to avoid static sparks.

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

8.1 CONTROL PARAMETERS

Occupational Exposure Limits

| Material | OSHA (PEL) | NIOSH (IDLH) | NIOSH (REL) | NIOSH (STEL) | ACGIH (TLV) | ACGIH (STEL) |
| --- | --- | --- | --- | --- | --- | --- |
| Citra-Solv | None Known |  |  |  | None Known |  |

Other Control Parameters	: Not available.

8.2 APPROPRIATE ENGINEERING CONTROL

Engineering Measures	: Use only with adequate ventilation. General ventilation (typically 10 air changes per hour) should be used. Ventilation rates should be matched to conditions. Local exhaust ventilation or an enclosed handling system may be necessary to control air contamination below that of the lowest OEL from the table above.

8.3 INDIVIDUAL PROTECTION MEASURES

Respiratory Protection    	: If ventilation is not sufficient, an approved respirator with organic vapor cartridge may be permissible under certain circumstances where airborne concentrations are expected to exceed occupational exposure limits. If respirators are needed, compliance with OSHA standard 29 CFR 1910.134 is necessary.

Skin Protection            	: Use protective clothing impervious to the ingredients listed in Section 2. Wear impervious gloves for prolonged contact. Use of aprons and boots is recommended.

Eye/Face Protection	: Safety glasses with side shields are recommended as a minimum for any type of industrial chemical handling. Where contact with this material could occur, chemical splash proof goggles are recommended.

Other Protective Equip.	: Safety showers and eye-wash stations should be available in the workplace near where the material will be used.

9. PHYSICAL AND CHEMICAL PROPERTIES

9.1 PHYSICAL PROPERTIES

| Boiling Point : 175-176°C (348-349°F) | Melting/Freezing Point : N/A |
| --- | --- |
| Flash Point : 49°C (124°F) | Percent Volatile : N/A |
| Explosive Limits : 0.6% lower – upper 7.0% (V) | Autoignition Temp, Liq. : 240°C |
| Flammability : Combustible Liquid | Relative Density (H2O=1) : 0.91 |
| Molecular Weight : Not Available | Weight : 7.60 lbs/gal |
| Vapor Pressure : 0.9 mm Hg @ 20°C | pH : NDA |
| Vapor Density : 4.75 (Air=1) | Evaporation Rate : 0.1 (n-Butyl Acetate =1) |
| Form : Liquid | Partition Coefficient : N/A |
| Viscosity : NDA | Refractive Index : N/A |
| Odor Threshold : N/A | Heat of Combustion (∆Hc) : N/A |
| Odor : Orange, citrus | Water Solubility : Soluble |
| Appearance/Color : Clear Amber | Decomposition Temp : N/A |

10. STABILITY AND REACTIVITY

10.1 REACTIVITY	: No dangerous reaction known under conditions of normal use.

10.2 CHEMICAL STABILITY	: This product is stable under normal conditions. Hazardous polymerization will not occur.

10.3 HAZARDOUS REACTIONS	: Under normal conditions of storage and use, hazardous reactions are not expected to occur.

10.4 CONDITIONS TO AVOID	: Contamination, elevated temperatures, static discharge and flames.

10.5 INCOMPATIBLE MATERIALS	: Strong oxidizing agents.

10.6 DECOMPOSITION PRODUCTS   	: Carbon dioxide, carbon monoxide and hydrocarbons.

11. TOXICOLOGICAL INFORMATION

11.1 ACUTE TOXICITY ESTIMATES (Mixture)

Acute Toxicity on Ingredients:

| Material | Oral LD₅₀ |  | Dermal LD₅₀ |  | Inhalation LC₅₀ |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Value | Species | Value | Species | Value | Time | Species |
| D-Limonene | >5000 mg/kg | rat | >5000 mg/kg | rabbit | NDA |  |  |

11.2 HEALTH HAZARD CLASSIFICATION

Carcinogen Data

| Calif Prop-65 | OSHA | NIOSH | ACGIH | NTP | IARC |
| --- | --- | --- | --- | --- | --- |
| No | No | No | No | No | No |

11.3 INFORMATION ON THE LIKELY ROUTES OF EXPOSURE/ENTRY

Routes for Exposure	: Skin contact, eye contact, inhalation and ingestion.

11.4 INFORMATION ON PHYSICAL, CHEMICAL & TOXICOLOGICAL EFFECTS

Symptoms of Exposure	: Refer to section 11.5 below

11.5 DELAYED & IMMEDIATE EFFECTS & ALSO CHRONIC EFFECTS FROM SHORT & LONG TERM EXPOSURE

Delayed Effects		: No known delayed effects.

Immediate Effects	:

Inhalation Irritation: Can cause moderate respiratory irritation that usually subsides after exposure ceases.

Skin Contact	: Can cause minor skin irritation and with repeated exposure, drying and chapping.

Eye Contact	: Vapors may irritate eyes. Direct or prolonged contact with eyes will be painful and irritating. Seek medical attention for prolonged irritation.

Ingestion Irritation	: If swallowed will cause irritation/corrosion of the gastrointestinal tract. Seek medical attention immediately.

Ingestion Toxicity	: Harmful if swallowed.

Acute Effects	: This product is considered to be a potential irritant. This product should not be used for any other purpose than the intended use in Sec 1.

Chronic Effects	: Not established.

12. ECOLOGICAL INFORMATION

12.1 ACUTE AQUATIC TOXICITY

| Material | FISH |  |  | INVERTEBRATES |  |  | AQUATIC PLANTS |  |  | MICROORGANISMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Type | Value | Period | Type | Value | Period | Type | Value | Period | Type | Value | Period |
| D-Limonene | LC50 | 0.61-0.79 mg/L | 96hr | EC50 | 69.60 mg/L | 48hr |  | NDA |  |  | NDA |  |

12.2 ECOLOGICAL DATA

| Material | PERSISTENCE & DEGRADABILITY |  |  |  | BIOACCUMULATIVE POTENTIAL |  | MOBILITY |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Persistence | BOD | COD | ThOD | Pow / Kow | BCF | Koc |
| NDA |  |  |  |  |  |  |  |

Very toxic to aquatic life with long lasting effects.

12.3 OTHER ADVERSE EFFECTS	: Avoid release into the environment.

13. DISPOSAL CONSIDERATIONS

13.1 WASTE TREATMENT METHODS

Waste Disposal	: Characteristics and waste stream classification can change with product use and location. It is the responsibility of the user to determine the proper storage, transportation, treatment and/or disposal methodologies for solvent materials and residues at the time of disposition. All waste must be disposed of in compliance with the respective national, federal, state and/or local regulations. Spent or discarded material may be a hazardous waste.

14. TRANSPORTATION INFORMATION

| Transportation Information | Ground Transportation (DOT) | Air Transportation (IATA) | Ocean Transportation (IMDG) |
| --- | --- | --- | --- |
| UN Number | COMBUSTIBLE LIQUID Not Regulated if <119gls | UN-1993 | UN-1993 |
| Proper Shipping Name |  | Flammable Liquids, N.O.S. (Terpene Hydrocarbons) | Flammable Liquids, N.O.S. (Terpene Hydrocarbons) |
| Hazard Class(es) |  | 3 | 3 |
| Packaging Group |  | III | III |
| Marine Pollutant |  | No | No |
| Hazard Label(s) |  |  |  |

15. REGULATORY INFORMATION

15.1 FEDERAL REGULATIONS

TSCA 8 (b) Inventory Status	: All components are listed or are exempt from listing on the Toxic Substances Control Act Inventory.

CERCLA/SARA Sec 313	: No 313 listed chemicals in this product.

SARA 311/312	: Fire Hazard

PA Right-to-Know Comp.	: D-Limonene

NJ Right-to-Know Comp.	: D-Limonene

16. OTHER INFORMATION

NFPA Hazard ID	: Health – 1        Flammability – 2       Instability – 0

HMIS Hazard ID	: Health – 1        Flammability – 2       Reactivity – 0

SDS Distribution	: The information in this document should be made available to all who may handle the product.

SDS Revision Number	: B

SDS Effective Date	: 11/01/2018

SDS Regulation	: Regulation 1907/2006/EC as amended by Regulation (EU) 453/2000.

In accordance with the provisions of Article 41, Industrial Safety & Health Act and OSHA Hazard Communication Standard (29 CFR 1910.1200).

Disclaimer	: The information is based on our current knowledge and is intended to describe the product for the purpose of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.